United Chemical&Textile

Soaping  Agent  G

This is a soaping agent suitable for all reactive dyed cellulosic substrates at low temperature. It reduces the consumption of water and energy, processing time, and overall process cost, in comparison to the conventional system,

[Properties]

- Appearance: white crystals

- PH value of 1% solution: 6～7

- Solubility: soluble in water

- Storage: Can be stored for one year when the product is enclosed in the proper container under the room temperature.

[Features]

- Promotes the removal of the unfixed dyes.

- Works on fabric at low temperature with good fastness.

- Can improve the soaping effect by using soft water

[Applications]

It is advisable to add 0.3-0.5g/l soaping agent G to the washing liquor. The goods are then treated at 85℃ for 15-20 minutes.

[Packaging]

Packed in Paper Bags and 25kgs Net Each

The information and recommendations presented here were compiled with the utmost care,but cannot be extended to cover every possible case.They are intended to serve as non-binding guidelines and must be adapted to the prevailing conditions.